# Overview

Let’s use a button as an input device on your Raspberry Pi. When the button is pressed, the General Purpose Input/Output (GPIO) connected to the button will change to low level (0V). You can detect the state of the GPIO using a Python script so that you can determine when the user has pressed the button.

# Step 1:  Connect It

Use the following wiring diagram to connect the sensor to your Raspberry Pi

Important Notes:

- The wire colors do not matter.  Just use whatever is available at your station.

- Make sure that you are plugging the wires into the correct location on both the Pi and the Sensor (especially the power and ground)

- The wires move around a lot, so make sure they are firmly in place when you are testing

# Step 2:  Code It

We have provided you with a Python code template that can run this sensor/device.  The code is located at:

/home/pi/Documents/rpi-iot-demos/button.py

Use the Geany editor to open this file (NOTE:  this will be slow, so be patient).  Once it is open, take some time to read the code to see how it works.

Once you are ready, do the following:

- Find the function called OnStateChange().

  - This function is called whenever the Raspberry Pi detects that the button has been pressed or depressed.

  - The buttonPin parameter is set to 0 when the button is pressed, and 1 when it is not pressed

def OnStateChange(buttonPin):

if buttonPin == 0:

print("Button Pressed")

if buttonPin == 1:

print("Button Depressed")

Modify this function so that it prints a message to the console whenever the button is pressed.

# Step 3:  Run / Test It

- To run the script, open a Linux console and navigate to the folder with your code

  - Helpful Linux Commands:
ls 			lists the contents of the directory
cd <folder_name> 	opens a folder (don’t type the < > characters)
cd .. 			exits the folder you are currently in
cd ~ 			takes you back to your home folder

- Run the python program by typing the following:

python3 button.py

- Press the button and see what happens!

  - If it doesn’t work, make sure that your wiring is correct and that you see lights on the sensor / device

- To exit the program, press Ctrl-C in the terminal