# Overview 
The active buzzer has built-in oscillating source, so it will beep as long as it is wired up, but it can only beep with fixed frequency.

# Step 1:  Connect It

Use the following wiring diagram to connect the sensor to your Raspberry Pi

Important Notes:

- The wire colors do not matter.  Just use whatever is available at your station.

- Make sure that you are plugging the wires into the correct location on both the Pi and the Sensor (especially the power and ground)

- The wires move around a lot, so make sure they are firmly in place when you are testing

# Step 2:  Code It

We have provided you with a Python code template that can run this sensor/device.  The code is located at:

/home/pi/Documents/rpi-iot-demos/active_buzzer.py

Use the Geany editor to open this file (NOTE:  this will be slow, so be patient).  Once it is open, take some time to read the code to see how it works.

Once you are ready, do the following:

- Find the function called loop().

  - This function is called whenever the program starts.  It runs forever.

def beep(time_in_seconds):

on()

time.sleep(time_in_seconds)

off()

time.sleep(time_in_seconds)

def loop():

while active == True:

beep(0.5)

Modify this function so that it beeps for 0.5 seconds.  To do this, you will make a new function called . . . wait for it . . . beep().

# Step 3:  Run / Test It

- To run the script, open a Linux console and navigate to the folder with your code

  - Helpful Linux Commands:
ls 			lists the contents of the directory
cd <folder_name> 	opens a folder (don’t type the < > characters)
cd .. 			exits the folder you are currently in
cd ~ 			takes you back to your home folder

- Run the python program by typing the following:

python3 active_buzzer.py

- Do you hear the beeping?

  - If it doesn’t work, make sure that your wiring is correct and that you see lights on the sensor / device

  - Can you modify this program so that it beeps an interesting pattern?

- To exit the program, press Ctrl-C in the terminal

# Step 4:  Make It Controllable

To make your program listen for MQTT messages, modify your main program to look like the following:

# --------------------------------------------

# Main Program Starts Here

# --------------------------------------------

if __name__ == '__main__':

setup(Buzzer)

try:

comm.connect(channels=[("jbcs/buzzer", 0)])

comm.listen(on_message)

print("Buzzer Running")

loop()

except KeyboardInterrupt:

destroy()

Now, create a new function above your main program.  This function will process any messages transmitted to your program.

# --------------------------------------------

# This is the code that will be run every time

# a message is received

# --------------------------------------------

def on_message(client, userdata, msg):

global active

print("Received:", msg)

if msg == "OFF":

active = False

elif msg == "ON":

active = True

# Step 4:  Make It Control Something

To make your program send MQTT messages, modify your main program to look like the following:

# --------------------------------------------

# Main Program Starts Here

# --------------------------------------------

if __name__ == '__main__':

comm.connect()

setup()

try:

print("Program Running.  Waiting for a Button Press")

loop()

except KeyboardInterrupt:

destroy()

Now, go to the OnStateChange() function you modified earlier.  You can use the comm.send function to send a message when the button is pressed, etc.

comm.send("CHANNEL NAME GOES HERE", "MESSAGE GOES HERE")