# Overview

A Hall Effect sensor is used to detect the presence of electromagnetic fields.  Electricity carried through a conductor will produce a magnetic field that varies with current, and a Hall sensor can be used to measure the current without interrupting the circuit. Typically, the sensor is integrated with a wound core or permanent magnet that surrounds the conductor to be measured.

# Step 1:  Connect It

Use the following wiring diagram to connect the sensor to your Raspberry Pi

Important Notes:

- The wire colors do not matter.  Just use whatever is available at your station.

- Make sure that you are plugging the wires into the correct location on both the Pi and the Sensor (especially the power and ground)

- The wires move around a lot, so make sure they are firmly in place when you are testing

# Step 2:  Code It

We have provided you with a Python code template that can run this sensor/device.  The code is located at:

/home/pi/Documents/rpi-iot-demos/hall_effect.py

Use the Geany editor to open this file (NOTE:  this will be slow, so be patient).  Once it is open, take some time to read the code to see how it works.

Once you are ready, do the following:

- Find the function called SensorChanged().

  - This function is called whenever a magnetic presence is detected or disappears

  - The current sensor reading is stored in sensorReading

def SensorChanged(sensorReading):

if sensorReading == 0:

print("Hall Effect 0")

elif sensorReading == 1:

print("Hall Effect 1")

Modify this function so that it prints a message to the screen whenever the magnetic field changes

# Step 3:  Run / Test It

- To run the script, open a Linux console and navigate to the folder with your code

  - Helpful Linux Commands:
ls 			lists the contents of the directory
cd <folder_name> 	opens a folder (don’t type the < > characters)
cd .. 			exits the folder you are currently in
cd ~ 			takes you back to your home folder

- Run the python program by typing the following:

python3 hall_effect.py

- Try placing a magnet near your sensor to see what happens.

  - If it doesn’t work, make sure that your wiring is correct and that you see lights on the sensor / device

  - Can you modify the program so that it prints the correct message when a magnet comes close and goes away?

- To exit the program, press Ctrl-C in the terminal