# Overview

A temperature sensor is a component that converts temperature readings into output signals. By material and component features, temperature sensors can be divided into two types: thermal resistor and thermocouple. For this lab, we will be working with the former. Essentially, the thermistor is a resistor whose resistance decreases with rising temperature. Using Python, we can periodically sample the resistance and convert it back into a temperature reading.  Since their resistance changes acutely with temperature changes, thermistors are the most sensitive temperature sensors.

# Step 1:  Connect It

Use the following wiring diagram to connect the sensor to your Raspberry Pi

Important Notes:

- The wire colors do not matter.  Just use whatever is available at your station.

- Make sure that you are plugging the wires into the correct location on both the Pi and the Sensor (especially the power and ground)

- The wires move around a lot, so make sure they are firmly in place when you are testing

# Step 2:  Code It

We have provided you with a Python code template that can run this sensor/device.  The code is located at:

/home/pi/Documents/rpi-iot-demos/temperature.py

Use the Geany editor to open this file (NOTE:  this will be slow, so be patient).  Once it is open, take some time to read the code to see how it works.

Once you are ready, do the following:

- Find the function called loop().

  - This function starts when the script runs, and runs forever

def loop():

while True:

temperature_in_celsius = read()

temperature_in_fahrenheit = temperature_in_celsius * (9/5) + 32

if temperature_in_celsius != None:

print ("Current temperature : %0.3f F" % temperature_in_fahrenheit)

Modify this function so that it gets the temperature (using the read() function), converts it to Fahrenheit, and prints it to the console.

# Step 3:  Run / Test It

- To run the script, open a Linux console and navigate to the folder with your code

  - Helpful Linux Commands:
ls 			lists the contents of the directory
cd <folder_name> 	opens a folder (don’t type the < > characters)
cd .. 			exits the folder you are currently in
cd ~ 			takes you back to your home folder

- Run the python program by typing the following:

python3 temperature.py

- Try heating up the temperature sensor to see what happens!

  - If it doesn’t work, make sure that your wiring is correct and that you see lights on the sensor / device

  - Can you modify the program so that it only prints once the temperature goes above/below some value?

- To exit the program, press Ctrl-C in the terminal

# Step 4:  Make It Control Something

To make your program send MQTT messages, modify your main program to look like the following:

# --------------------------------------------

# Main Program Starts Here

# --------------------------------------------

if __name__ == '__main__':

comm.connect()

setup()

try:

setup()

print("Temperature sensor started.")

loop()

except KeyboardInterrupt:

destroy()

Now, go to the loop() function you modified earlier.  You can use the comm.send function to send a message when the temperature is above/below a certain value, etc.

comm.send("CHANNEL NAME GOES HERE", "MESSAGE GOES HERE")